- Introduction

This documentary note describes the USB API Library for Performax Devices on Apple Mac and the procedure for setting up the library and using it.

- USB API Library for Performax device on Mac

The USB API library for Performax device on Mac, ‘PerformaxDriverMac.dylib’ is built using Xcode Version 4.6 on MacBook with Mac OS X Version 10.7.5.The project type of the API library is a Cocoa Library (dynamic) project. It comprises of the following files:

- PerformaxDriverMac.h

- PerformaxDriverMac.m

- libusb.h (part of the USB driver described below)

  - API Function Listing

The following functions are available to the client application which uses the API library:

- fnPerformaxComGetNumDevices – Returns the number of Performax devices that are connected to the Mac.

- fnPerformaxComGetProductString – Returns a string which identifies the Performax product. This string can be used to distinguish devices when multiple of them is connected to a single computer.

- fnPerformaxComOpen – Returns a handle to a Performax device. This handle can be used to perform other actions on this device.

- fnPerformaxComSendRecv – Used to read data from / write data to a Performax device.

- fnPerformaxComFlush – Used to flush a Performax device.

- fnPerformaxComClose – Used to close a Performax device.

- fnPerformaxComSetTimeouts – Used to set read / set timeouts.

- USB driver

The above API library depends on the USB driver, “libusb-1.0.0.dylib”. The version of the USB driver used here is “libusb-1.0.9”. This package can be downloaded from www.libusb.org, built and installed on MacBook. The header file ”libusb.h” (which is a part of the “libusb-1.0.9” USB driver package) is needed to build the API library.

- GUI Test Application for Performax device on Mac

The GUI test application for Performax device on Mac is built using Xcode Version 4.6 on MacBook with Mac OS X Version 10.7.5. The project type of the GUI test application is a Cocoa application project. It mainly comprises of the following files:

- PerformaxController.h

- PerformaxController.m

- PerformaxDriverMac.h (part of the API library described above)

- MainMenu.xib

- ArcusAppDelegate.m

The following buttons are available in the GUI test application:

- Search – When this button is clicked, the application gets the number of Performax devices that are available and displays it in a text box.

- Open Device –After selecting a Performax device number from the combo box, when this button is clicked, the application gets and stores a handle to the Performax device.

- Close Device – After selecting a Performax device number from the combo box, when this button is clicked, the application closes communication to the Performax device.

- Flush Device – After selecting a Performax device number from the combo box, when this button is clicked, the application flushes the Performax device.

- Set Timeouts –After giving appropriate values for read / write timeouts in the provided text boxes, when this button is clicked, the application sets the read / write timeouts for communication with Performax devices.

- Get Product String - After selecting a Performax device number from the combo box, when this button is clicked, the application gets a string which identifies the Performax product and displays it in a text box.

- Get Product ID - After selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to the Performax device to getthe product ID string and displays it in a text box.

- Get Device Number - After selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to the Performax device to get the device number string and displays it in a text box.

- Enable Device – After selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to enable the Performax device.

- Set Low speed - After giving appropriate value for Low speed in the provided text box and selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to the Performax device to set the Low speed value.

- Set High speed - After giving appropriate value for High speed in the provided text box and selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to the Performax device to set the High speed value.

- Set Polarity - After giving appropriate value for Polarity in the provided text box and selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to the Performax device to set the Polarity value.

- Set Acceleration - After giving appropriate value for Acceleration in the provided text box and selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to the Performax device to set the Acceleration value.

- Move Motor - After giving appropriate value for distance in the provided text box and selecting a Performax device number from the combo box, when this button is clicked, the application sends a command to the Performax device to move the motor by that distance.

- Execute –This button can be used to execute any command (get/set) on the Performax device. After giving an appropriate request in the provided text box and selecting a Performax device number from the combo box, when this button is clicked, the application sends the command to the Performax device and displays the response from the Performax device in a text box.

  - GUI Test Application Dependencies

The GUI test application depends on the API library, “PerformaxDriverMac.dylib”. The header file ”PerformaxDriverMac.h” of the API library is needed to build the GUI test application.

- Installation of the GUI Test Applicationand library files

The installer package ‘Performax USB API Library Setup’ copies the library files “libusb-1.0.0.dylib” and “PerformaxDriverMac.dylib” in ‘/usr/local/lib’ folder of the Mac, and the GUI test application ‘TestAppPerformax’ in the ‘Applications’ folder.